## SECTION A — ANALYTICAL MCQs

1. Same ISA, different processor speed. Best reason?
A. ISA fixes software behavior, not internal design
B. ISA fixes every circuit
C. ISA replaces control
D. ISA removes memory

2. A CPU has an ALU but no PC. What fails first?
A. Arithmetic
B. Instruction fetch
C. Register naming
D. Cache replacement

3. add $t0, 0($s0), $t1 is wrong because:
A. MIPS arithmetic uses registers, not memory operands
B. $s0 cannot hold addresses
C. add cannot use $t0
D. Memory cannot store integers

4. addi needs extra datapath support mainly because it uses:
A. Immediate operand
B. Data memory write
C. Jump target
D. Dirty bit

5. In lw, correct address is computed but register gets address, not data. Fault?
A. Write-back mux
B. PC adder
C. Branch comparator
D. Instruction memory

6. In sw, enabling RegWrite is dangerous because:
A. Store should write memory, not register
B. Store cannot use ALU
C. Store must branch
D. Store reads no register

7. A branch comparison works, but PC never changes. Suspect:
A. PC selection logic
B. Data memory size
C. Cache block offset
D. Assembler

8. j differs from beq because j is:
A. Unconditional
B. Memory-based
C. Register arithmetic
D. Cache-dependent

9. jr $ra needs a datapath path from:
A. Register file to PC
B. Data memory to ALU
C. Cache tag to PC
D. Immediate to memory

10. CPI = 1 but slow CPU. Why?
A. Long clock due to slowest instruction
B. No instruction executes
C. ALU is unused
D. ISA disappears

11. Reusing loop counter shows:
A. Temporal locality
B. Spatial locality
C. Structural hazard
D. Control signal

12. Reading array elements in order shows:
A. Spatial locality
B. Dirty block
C. RAW hazard
D. Exception

13. Two useful blocks repeatedly replace each other. Main issue?
A. Conflict miss
B. Compulsory miss only
C. Illegal instruction
D. Write-back success

14. Fully associative cache reduces conflicts but increases:
A. Search complexity
B. Instruction length
C. ALU operation count
D. PC value

15. Write-through is simpler because:
A. Memory is updated immediately
B. Dirty bit is always required
C. It avoids all misses
D. It removes cache

### SECTION B — SHORT QUESTIONS 5 × 3 = 15

1. Why is ISA called a bridge between software and hardware?

2. Explain why “CPU = ALU” is an incomplete statement.

3. Compare lw and sw using only data direction and write enables.

4. Why can one datapath execute many instructions only with correct control?

5. Why can direct-mapped cache be fast but still inefficient?

6. Why does pipelining improve throughput but not necessarily latency?

7. Identify the hazard type only:

- One memory is needed by IF and MEM.

- add writes $t0; next instruction reads $t0.

- Branch direction is not known yet.

## SECTION C (answer any three sub section of each )

## Q1. Datapath and Control Diagnosis — 15 Marks

A student builds a processor for:

add, addi, lw, sw, beq, j

He says: “The ALU can do the main work, so control is secondary.”

Answer:

- Why is the ALU not enough?

- Give one datapath requirement for each: add, addi, lw, sw, beq, j.

- Explain how wrong control can make a correct datapath produce wrong output.

- Diagnose two faults:

  - lw writes address instead of data.

  - sw changes a register unexpectedly.

- Why should datapath design start from the ISA?

## Q2. Memory and Cache Judgment — 15 Marks

A lab upgrades the processor but ignores memory and cache design. Some programs remain slow.

Answer:

- Why can memory still limit a fast processor?

- Explain memory hierarchy in terms of speed, size, and cost.

- Compare direct-mapped, fully associative, and set-associative cache analytically.

- Identify locality in:

  - Loop counter reused many times.

  - Array scanned sequentially.

- Why does write-back need a dirty bit?

## Q3. Pipeline Hazard Analysis — 15 Marks

A five-stage pipeline uses:

IF → ID → EX → MEM → WB

A student says: “Pipelining is bad because hazards appear.”

Answer:

- Defend pipelining despite hazards.

- Why are pipeline registers needed?

- Analyze the hazard:

add $t0, $t1, $t2
sub $t3, $t0, $t4

- A single memory is used for both instruction fetch and data access. What hazard occurs and how can hardware reduce it?