- INSTRUCTION SET ARCHITECTURE (ISA)

- Descriptive Theoretical Notes for Students

- 🧭 1. Understanding Instruction Set Architecture (ISA)

Instruction Set Architecture (ISA) is one of the most fundamental concepts in computer systems. It defines the way a computer understands and executes instructions. In simple terms, ISA acts as a communication bridge between software and hardware.

Every program that we write in a high-level language (such as C, Java, or Python) is eventually converted into a set of instructions that the processor can understand. These instructions are defined by the ISA.

Thus, ISA determines:

- What operations a computer can perform

- How instructions are represented

- How data is accessed and manipulated

- 🎯 Conceptual Understanding

A useful way to understand ISA is through analogy:

- Humans communicate using languages like English or Urdu

- Computers communicate using ISA

Therefore, ISA can be considered the “language of the processor.”

Without ISA, software would have no standardized way to interact with hardware, making computation impossible.

- ⚙️ 2. Instruction Execution Cycle

Every instruction executed by the CPU follows a systematic process known as the Instruction Execution Cycle. This cycle is repeated continuously while a program runs.

- 🔄 Detailed Steps of Execution

- 1. Fetch

The CPU retrieves the instruction from memory using the Program Counter (PC). This step ensures that the correct instruction is brought into the processor.

- 2. Decode

Once the instruction is fetched, the CPU decodes it. This means:

- Identifying the operation (e.g., ADD, SUB)

- Identifying the operands involved

- 3. Operand Fetch

The required data is fetched from registers or memory. For example, if the instruction requires values stored in registers R2 and R3, the CPU retrieves them.

- 4. Execute

The CPU performs the required operation using its Arithmetic Logic Unit (ALU). For example, it may add two numbers.

- 5. Store Result

The result of the operation is stored back into a register or memory location.

- 6. Next Instruction

The CPU updates the Program Counter to move to the next instruction and repeats the cycle.

- 🎯 Important Insight

This entire process occurs extremely fast—millions or even billions of times per second—making modern computing possible.

- 🧠 3. Key Design Decisions in ISA

Designing an ISA is a complex task that involves several important decisions.

- 🔹 Operations

The ISA must define the set of operations the processor can perform, such as:

- Arithmetic operations (addition, subtraction)

- Logical operations (AND, OR)

- Data transfer operations (LOAD, STORE)

The number and type of operations directly affect the flexibility and complexity of the system.

- 🔹 Operands

Operands are the data on which operations are performed. These can be:

- Registers (fast storage inside CPU)

- Memory locations (larger but slower storage)

- Immediate values (constants embedded in instructions)

- 🔹 Instruction Format

ISA defines how instructions are structured in binary form. Each instruction consists of:

- Opcode (operation code)

- Operand fields

The format determines how the CPU interprets the instruction.

- 💡 Key Concept

Binary instructions (machine code) must be precisely interpretable, otherwise the CPU cannot execute them correctly.

- 📏 4. Instruction Length

Instruction length refers to the number of bits used to represent an instruction.

- 🔹 Fixed-Length Instructions

In architectures like MIPS, all instructions are of equal size (typically 32 bits).
This simplifies:

- Instruction decoding

- Pipelining

- Hardware design

- 🔹 Variable-Length Instructions

In architectures like x86, instructions can have different lengths.
This allows:

- More flexibility

- Compact programs

However, it increases complexity in decoding.

- 🎯 Insight

Modern RISC architectures prefer fixed-length instructions for better performance.

- 🧱 5. Instruction Formats

Instruction format defines how bits within an instruction are organized.

- 🔹 Common Formats in MIPS

- R-Type (Register Format)

Used for operations involving registers only.

Example:
ADD R1, R2, R3
→ R1 = R2 + R3

- I-Type (Immediate Format)

Used for operations involving memory or constants.

Example:
LOAD R1, 100(R2)

- J-Type (Jump Format)

Used for control flow instructions such as jumps.

Example:
J 1000

- 🎯 Conceptual Insight

Fewer instruction formats make the processor:

- Easier to design

- Faster to execute instructions

- 📍 6. Data Storage: Registers vs Memory

Data used by the CPU can be stored in two main places:

- 🔹 Registers

- Located inside the CPU

- Extremely fast

- Limited in number

- 🔹 Memory

- Located outside the CPU

- Larger storage capacity

- Slower compared to registers

- 🎯 Key Principle

Efficient programs try to use registers as much as possible to improve performance.

- ⚡ 7. Load-Store Architecture

Modern RISC systems use a load-store architecture, where:

👉 Only LOAD and STORE instructions access memory
👉 All other operations use registers only

- Example:

Correct:

- LOAD R1, M[100]

- ADD R2, R1, R3

Incorrect:

- ADD R1, R2, M[100]

- 🎯 Why Load-Store?

- Simplifies hardware design

- Enables faster execution

- Supports pipelining

- 🔢 8. Number of Operands

Different ISA designs use different numbers of operands.

- 🔹 Types of Architectures

- Stack-Based (0 operands)

Uses implicit operands from stack

- Accumulator-Based (1 operand)

Uses a special accumulator register

- Register-Memory (2 operands)

Operands may come from memory or registers

- Load-Store (3 operands)

Example:
ADD R1, R2, R3

- 🎯 Insight

More operands:

- Reduce number of instructions

- Increase instruction complexity

- 📍 9. Addressing Modes

Addressing modes define how the CPU accesses operands.

- 🔑 Common Addressing Modes

- Immediate Addressing

Value is part of instruction
Example: MOV R1, #5

- Register Addressing

Operand is in register
Example: ADD R1, R2

- Direct Addressing

Instruction specifies memory address

- Indirect Addressing

Register contains memory address

- Base + Displacement

Address = base register + offset

- 🎯 Key Insight

Most programs use simple addressing modes, making them highly efficient.

- 🧠 10. Memory Organization

Memory is structured as a large array of bytes.

- 🔹 Key Concepts

- Each memory location has an address

- Byte = 8 bits

- Word (in MIPS) = 32 bits

- 🔹 Alignment

Words are stored at addresses like:
0, 4, 8, 12...

This ensures faster memory access and efficient processing.

- 🔄 11. Types of Instructions

- 🔹 Arithmetic Instructions

Perform mathematical operations (ADD, SUB)

- 🔹 Logical Instructions

Perform bitwise operations (AND, OR)

- 🔹 Data Transfer Instructions

Move data (LOAD, STORE)

- 🔹 Control Instructions

Change program flow (JUMP, BRANCH)

- 🔀 12. Control Flow Instructions

Control flow allows decision-making in programs.

- 🔹 Branch Instructions

Example:
BEQ R1, R2, addr

If condition is true → jump

- 🔹 Jump Instructions

Used for unconditional jumps

- 🎯 Important Concept

Branches often use relative addressing, making them efficient.

- ⚔️ 13. RISC vs CISC Architectures

- 🧠 RISC (Reduced Instruction Set Computer)

- Simple instructions

- Fixed instruction length

- Fast execution

- 🧠 CISC (Complex Instruction Set Computer)

- Complex instructions

- Variable instruction length

- Direct memory access

- ⚖️ Comparison

| Feature | RISC | CISC |
| --- | --- | --- |
| Speed | High | Lower |
| Complexity | Low | High |
| Flexibility | Moderate | High |

- 🎯 Final Insight

Modern processors often combine features of both RISC and CISC.

- 🧾 14. Final Summary

Instruction Set Architecture (ISA) is the foundation of computer operation. It defines how instructions are executed and how software interacts with hardware.

Key takeaways:

- ISA is the language of the CPU

- Instruction execution follows a fixed cycle

- Registers are faster than memory

- Load-store architecture improves performance

- Addressing modes determine data access

- RISC and CISC represent two design philosophies

- ✍️ 15. Examination Preparation

Students should be able to:

- Define ISA and explain its importance

- Describe instruction execution cycle in detail

- Explain different instruction formats

- Compare RISC and CISC architectures

- Analyze addressing modes with examples