## Memory Systems

### Descriptive Theoretical Notes for Students

# 1. Introduction to Memory Systems

A computer is not useful unless it can store instructions and data and retrieve them when needed. The processor performs calculations very quickly, but it constantly depends on memory for:

- program instructions,

- input data,

- intermediate values,

- and final results.

This makes memory one of the most important parts of a computer system.

In theory, we may think of memory as a simple place where everything is stored. In practice, however, memory is a major performance challenge. The processor is extremely fast, but memory is much slower. This speed difference creates what is often called the processor-memory gap.

# 2. Why Memory Matters

The CPU can execute operations in a very short time, but many instructions require memory access. For example:

- instruction fetch requires reading from instruction memory,

- lw reads data from memory,

- sw writes data to memory,

- arrays, structures, and program variables all reside in memory.

If memory is too slow, then even a fast processor must wait. This means that a system’s performance depends not only on how powerful the CPU is, but also on how efficiently memory is organized.

# 3. The Basic Problem: CPU is Fast, Memory is Slow

Modern processors operate at extremely high speeds, often billions of cycles per second. Main memory, however, is much slower than the CPU. If the processor had to directly access slow memory for every operation, most of its time would be wasted waiting.

This is why memory systems are carefully designed in layers. The goal is to make the computer appear to have:

- the speed of small memory

- and the capacity of large memory

at the same time.

# 4. Memory Hierarchy

To solve the speed problem, computers use a memory hierarchy.

A memory hierarchy is an arrangement of different storage levels, where:

- the smallest and fastest memory is closest to the CPU,

- and the largest and slowest memory is farther away.

## Typical Memory Hierarchy

From fastest to slowest:

- Registers

- Cache Memory

- Main Memory (RAM)

- Secondary Storage (SSD/HDD)

- Tertiary or Backup Storage (optional in some systems)

## Main Idea of Memory Hierarchy

As we move downward in the hierarchy:

- speed decreases

- capacity increases

- cost per bit decreases

So:

- registers are very fast but tiny,

- storage devices are huge but much slower.

# 5. Registers

Registers are the fastest storage locations in a computer. They are located inside the CPU and hold the values that are currently being used.

### Characteristics of Registers

- extremely fast,

- very limited in number,

- directly accessed by CPU instructions,

- used for temporary values and arithmetic operations.

### Why they are important

Arithmetic operations usually happen on register values, not directly on memory values. This is why load-store architectures move data into registers before computation.

# 6. Cache Memory

Cache is a small, fast memory placed between the CPU and main memory. Its job is to keep copies of the data and instructions that are likely to be used again soon.

### Simple definition

Cache memory is a high-speed temporary storage area that reduces average memory access time.

### Why cache exists

If the CPU had to fetch everything from RAM all the time, performance would be very poor. Cache stores recently used or nearby data so that the CPU can access it quickly.

## Example

Suppose a program is repeatedly using elements of an array. Instead of going to RAM every time, the needed block may already be in cache. This saves time.

# 7. Main Memory (RAM)

Main memory, or RAM, stores programs and data that are currently in use. It is larger than cache but slower.

### Characteristics

- directly accessible by the processor,

- volatile storage,

- holds active programs and data,

- slower than registers and cache,

- faster than secondary storage.

RAM is the working memory of the computer. When you open a program, it is loaded from storage into RAM so the CPU can work with it more efficiently.

# 8. Secondary Storage

Secondary storage includes devices such as:

- SSDs,

- HDDs,

- flash drives.

It is used for long-term data storage.

### Characteristics

- non-volatile,

- much larger than RAM,

- much slower than RAM,

- used to permanently store files, programs, and the operating system.

Programs are stored here when the computer is off. When a program runs, it must be moved into RAM.

# 9. The Principle of Locality

The memory hierarchy works because of a very important idea called locality of reference.

Programs do not access memory randomly all the time. Instead, they tend to access certain memory locations repeatedly or access nearby locations.

There are two major types of locality:

## 9.1 Temporal Locality

If a program uses a piece of data now, it is likely to use the same data again soon.

### Example

A loop variable used repeatedly in a loop.

## 9.2 Spatial Locality

If a program uses one memory location, it is likely to use nearby locations soon.

### Example

Accessing consecutive elements of an array.

## Why locality matters

Cache relies on locality. If recently used data or nearby data is likely to be used again, keeping it in fast memory makes the whole system much faster.

# 10. Cache Basics

Cache does not usually transfer just one byte or one word at a time. Instead, it transfers a block (also called a line) of memory.

### Important cache terms

- Cache block / line: unit of transfer between memory and cache

- Hit: required data is found in cache

- Miss: required data is not in cache

- Hit rate: percentage of accesses found in cache

- Miss rate: percentage of accesses not found in cache

- Miss penalty: extra time needed to fetch data from lower memory level

## Hit

When the CPU asks for data and that data is already in cache, it is called a cache hit.

### Result

Fast access, better performance.

## Miss

When the CPU asks for data and it is not in cache, it is called a cache miss.

### Result

The block must be fetched from main memory, which takes longer.

# 11. Average Memory Access Time (AMAT)

A simple way to think about cache performance is:

Average Memory Access Time = Hit Time + (Miss Rate × Miss Penalty)

### Meaning

- Hit Time = time to access cache

- Miss Rate = how often cache misses happen

- Miss Penalty = extra delay when data must come from lower memory

This formula shows that cache performance depends not only on cache speed, but also on how often misses occur and how expensive those misses are.

# 12. How Cache Works

When the CPU requests data:

- the cache checks whether the block is already present,

- if yes, it supplies the data immediately,

- if not, the block is fetched from main memory,

- the cache stores the block,

- the CPU then receives the needed data.

This process makes the system efficient because many future accesses may become hits.

# 13. Cache Mapping Techniques

A major design issue is this:

Where should a block from main memory be placed in cache?

Different cache organizations answer this differently.

## 13.1 Direct-Mapped Cache

In a direct-mapped cache, each memory block can go to only one specific cache line.

### Advantages

- simple,

- fast,

- easy to implement.

### Disadvantages

- higher chance of conflict misses,

- two frequently used blocks may compete for the same line.

### Simple idea

Each memory block has exactly one “allowed slot.”

## 13.2 Fully Associative Cache

In a fully associative cache, a memory block can go to any cache line.

### Advantages

- very flexible,

- fewer conflict misses.

### Disadvantages

- hardware is more complex,

- slower and more expensive to search.

### Simple idea

A block may be placed anywhere in cache.

## 13.3 Set-Associative Cache

A set-associative cache is a compromise between the two.

Cache is divided into sets, and a block can be placed in any line within its set.

### Advantages

- fewer conflicts than direct-mapped,

- cheaper than fully associative.

### Disadvantages

- more complex than direct-mapped.

### Example

In a 2-way set-associative cache, each set has 2 lines, and a block can go into either of them.

# 14. Types of Cache Misses

Cache misses can happen for different reasons.

## 14.1 Compulsory Miss

This happens the first time data is accessed, because it has never been loaded before.

### Also called

Cold-start miss.

## 14.2 Capacity Miss

This happens when the cache is too small to hold all the needed data.

## 14.3 Conflict Miss

This happens when multiple blocks compete for the same cache location, especially in direct-mapped or low-associativity caches.

# 15. Cache Replacement Policies

If the cache is full and a new block must be brought in, one old block has to be removed.

This decision is made by the replacement policy.

## Common replacement policies

### Least Recently Used (LRU)

Replace the block that has not been used for the longest time.

### First In, First Out (FIFO)

Replace the oldest block.

### Random

Replace a randomly selected block.

## Why replacement policy matters

A good policy keeps useful data in cache longer and reduces misses.

# 16. Write Policies in Cache

When the CPU writes data, cache designers must decide what happens to main memory.

There are two major policies:

## 16.1 Write-Through

Every write updates:

- the cache,

- and main memory immediately.

### Advantages

- memory always stays updated,

- simpler consistency.

### Disadvantages

- more memory traffic,

- slower writes.

## 16.2 Write-Back

The write updates cache first. Main memory is updated later, only when the block is removed.

### Advantages

- fewer memory writes,

- better performance.

### Disadvantages

- more complex,

- memory may not always have the newest value instantly.

# 17. Multi-Level Cache

Modern processors often use more than one cache level:

- L1 Cache: very small and very fast

- L2 Cache: larger but slower than L1

- L3 Cache: even larger, shared in many systems

This multi-level approach improves performance by keeping frequently used data very close to the processor while still providing larger fast-storage layers before RAM.

# 18. Instruction Cache and Data Cache

Some systems separate cache into:

- Instruction Cache (I-Cache) for instructions

- Data Cache (D-Cache) for data

### Why separate them?

Because instruction fetch and data access may happen at the same time, especially in pipelined systems.

This separation improves parallelism and avoids conflicts between instruction and data accesses.

# 19. Main Memory Organization

Main memory is usually organized as a large array of bytes, addressed by location. It stores:

- instructions,

- variables,

- arrays,

- stack,

- heap,

- and operating system data.

The CPU usually does not access the disk directly while executing instructions. Instead:

- data is loaded from storage into RAM,

- then RAM interacts with cache and CPU.

# 20. Virtual Memory (Introductory View)

Virtual memory is a technique that gives each program the illusion of having a large, continuous memory space.

It allows:

- running programs larger than physical memory,

- memory protection,

- isolation between processes.

The operating system and hardware cooperate to map virtual addresses to physical addresses.

At an introductory level, students should understand that virtual memory:

- improves flexibility,

- supports protection,

- and uses disk as an extension of RAM when necessary.

# 21. Why Memory Systems Improve Performance

A good memory system improves performance because:

- most accesses are served from fast nearby storage,

- cache reduces waiting time,

- locality makes repeated access efficient,

- and layered memory balances speed, size, and cost.

Without memory hierarchy, processor performance would be severely limited.

# 22. Real-World Insight

A powerful processor alone does not guarantee a fast computer. If memory is slow or badly organized:

- programs load slowly,

- data access becomes delayed,

- execution stalls become common,

- and the CPU remains underutilized.

This is why computer architects pay close attention to:

- cache design,

- memory latency,

- bandwidth,

- hierarchy structure,

- and memory management.

# 23. Common Student Confusions

### Confusion 1: Cache and RAM are the same

They are not the same. Cache is smaller and much faster. RAM is larger but slower.

### Confusion 2: More RAM means faster CPU

Not always. More RAM helps capacity, but CPU speed also depends heavily on cache and processor design.

### Confusion 3: CPI = 1 means always fast

Not necessarily. A processor may have CPI = 1 but still be slow if each cycle is long because memory access is slow. This directly connects to the previous lecture’s discussion of slow single-cycle performance.