# Lecture-wise Simplified Notes

## Topic: Processor Design – Datapath and Control

## Slide 1 – Title Slide

This slide introduces the topic: Processor Design: Datapath and Control. It tells students that the lecture is about how a processor is actually built so that it can execute instructions, not just what instructions mean in theory.

### Simple note for students

Until now, you may have studied instructions like add, lw, and beq as written commands. In this lecture, you study the hardware that makes those instructions work inside the CPU.

## Slide 2 – Where We Are in This Course Right Now

This slide explains the course progression:

- earlier, students learned what architecture is,

- what instructions exist,

- and how ALU operations work,

- now, they move to full processor design,

- later, they will study memory systems.

### Simplified explanation

This lecture is a bridge between:

- instruction understanding, and

- processor implementation.

The ALU alone is not enough. A complete processor must also fetch instructions, decode them, and run them correctly.

## Slide 3 – This Unit: Processor Design

This slide lists the full scope of the unit:

- datapath components and timing,

- registers and register files,

- RAMs,

- mapping an ISA to datapath,

- control,

- exceptions.

### Simplified explanation

This whole unit answers one big question:

How do we turn an ISA into real hardware?

Students should understand that processor design is not one component; it is a combination of storage, computation, data movement, timing, and decision-making.

## Slide 4 – Readings

The slide recommends Patterson and Hennessy Chapter 4, Sections 4.1–4.4.

### Simplified explanation

This is a reading support slide. The message is:

- class time is limited,

- textbook examples are important,

- students should read carefully because the topic is highly visual and cumulative.

## Slide 5 – So You Have an ALU…

This slide makes a major conceptual point: a processor is a finite state machine (FSM) that interprets an ISA, and the ALU is only one part of the entire execution process. It mentions the need to fetch instructions, read/write registers, support loads/stores, and support branches.

### Simplified explanation

Students often think:

CPU = ALU

That is incomplete.

The ALU can calculate, but a processor must also:

- know which instruction to run,

- get the operands,

- decide where the result goes,

- access memory,

- and sometimes change control flow.

### Class emphasis

A processor is not just a calculator. It is a coordinated machine.

## Slide 6 – Datapath and Control

This slide defines:

- Datapath as registers, memories, ALUs,

- Control as deciding which registers read/write and what ALU does,

- Fetch as getting the instruction and translating it into control,

- and the processor cycle as Fetch → Decode → Execute.

### Simplified explanation

This is one of the most important slides in the lecture.

### Key idea

- Datapath = hardware that carries and processes data

- Control = signals that tell hardware what to do

### Easy analogy

- Datapath = road system and machines

- Control = traffic police or manager

Without datapath, nothing can move.
Without control, the datapath does not know what to do.

## Slide 7 – Building a Processor for an ISA

This slide says fetch is straightforward:

- PC holds the next address,

- instruction memory returns the instruction at that address,

- and datapath design starts by looking at the ISA and ensuring every instruction can be implemented.

### Simplified explanation

Processor design starts from the instruction set.

### Design logic

- Look at the instruction types.

- Ask what hardware each instruction needs.

- Build datapath paths to support them.

This is a systematic design method.

## Slide 8 – Datapath for MIPS ISA

This slide limits the focus to a small set of MIPS-like instructions:

- add

- addi

- lw

- sw

- beq

- j

It says these are enough because most other instructions are similar from a datapath point of view.

### Simplified explanation

Instead of designing for the entire ISA at once, the lecture uses a small “representative” set of instructions.

### Why this is good for students

Once you understand these six instructions, you can usually extend the same reasoning to many other instructions.

## Slide 9 – Review: A Register

This slide reviews a register as a D flip-flop array with a shared clock and write enable.

### Simplified explanation

A register is a very small, very fast storage unit inside the CPU.

### What students should remember

- It stores one value.

- It updates only when allowed.

- The write enable decides whether a new value is written.

This is the basic building block for larger structures like register files and the PC.

## Slide 10 – Uses of Registers

This slide highlights that a single register is useful for things like the Program Counter (PC) and other internal values that are not part of the ISA-visible registers.

### Simplified explanation

Not every register in a processor is visible to the programmer.

### Examples

- PC holds the address of the next instruction.

- Other internal registers may hold temporary values.

So processors use registers for both:

- architectural state,

- and internal control/temporary purposes.

## Slide 11 – What About the ISA Registers?

This slide introduces the register file:

- it contains the visible ISA registers,

- has two read ports and one write port,

- and allows parallel register access. It also explains buses and port widths.

### Simplified explanation

A register file is like a bank of registers.

### Why two read ports?

Because many instructions need two source operands.
Example: add $d, $s1, $s2

### Why one write port?

Because the result usually goes to one destination register.

### Student memory point

The register file supports:

- reading two registers at once,

- writing one result back.

That is exactly what many arithmetic instructions need.

## Slide 12 – Register File With Tri-State Read Ports

This slide shows a more detailed internal implementation diagram of the register file using tri-state read ports.

### Simplified explanation

This is a lower-level hardware view of the register file.

### What students really need to understand

Do not get lost in the detailed wiring. Focus on the big idea:

- the register file is designed so the CPU can select registers by number,

- read their contents,

- and write results efficiently.

For BS-level study, the concept of “two reads and one write” is more important than memorizing every internal wire.

## Slide 13 – Another Useful Component: Memory

This slide defines memory as the place where instructions and data reside. It describes one read/write port, one address bus, and separate input/output data buses.

### Simplified explanation

Memory is larger than registers but slower.

### In this lecture, memory appears in two forms

- Instruction memory: holds the program code

- Data memory: holds data values used by instructions

### Student takeaway

Registers are fast and small.
Memory is large and slower.

## Slide 14 – Dramatis Personae

This slide introduces the main components used in the datapath:

- PC

- memory

- register file

- ALU

- shift-left-by-2 unit

- adders

- sign extender

- mux

- AND gate

- zero extender.

### Simplified explanation

This is the vocabulary slide for processor hardware.

### Quick meanings

- Adder: adds values

- Shift-left-by-2: used in address calculations for branch/jump

- Sign extender: extends signed immediate values

- Zero extender: extends unsigned values

- Mux: chooses one of multiple inputs

- AND gate: helps control branch decisions

### Best way to revise

Students should learn each component’s job, not just its symbol.

## Slide 15 – Let’s Build a MIPS-like Datapath

This is a transition slide announcing the actual construction of the datapath.

### Simplified explanation

Now the lecture stops defining pieces and starts combining them into a working processor.

## Slide 16 – Start With Fetch

This slide shows the first step:

- PC

- instruction memory

- a +4 incrementer for the next instruction address. It also asks why the increment is by 4.

### Simplified explanation

Fetch is the process of getting the next instruction.

### Why PC + 4?

Because MIPS instructions are 32 bits = 4 bytes.
So each next instruction begins 4 bytes later.

### Student takeaway

The processor starts by:

- reading the instruction at the current PC,

- preparing the default next PC.

## Slide 17 – First Instruction: add $rd, $rs, $rt

This slide adds:

- register file

- ALU

to support register-to-register addition.

### Simplified explanation

To execute add, the CPU must:

- fetch the instruction,

- read two source registers,

- send them to the ALU,

- perform addition,

- write result into destination register.

### Big lesson

The datapath must physically connect:

- instruction fields,

- register file ports,

- ALU inputs,

- write-back path.

## Slide 18 – Second Instruction: addi $rt, $rs, imm

This slide adds:

- sign extension,

- a mux into ALU input B,

- and support for choosing rt as destination.

### Simplified explanation

addi is like add, but the second operand is an immediate constant, not a second register.

### Why sign extension?

Because the immediate field is smaller and must be extended to full ALU width.

### Why the mux?

Because now ALU input B can come from:

- register value, or

- immediate value.

## Slide 19 – Third Instruction: lw $rt, imm($rs)

This slide adds:

- data memory,

- ALU address computation,

- a mux to select whether write-back comes from ALU result or memory output.

### Simplified explanation

For lw, the CPU:

- takes base register rs,

- adds the sign-extended immediate,

- gets a memory address,

- reads data from memory,

- writes that data into register rt.

### Main point

lw needs both:

- address calculation,

- and memory reading.

## Slide 20 – Fourth Instruction: sw $rt, imm($rs)

This slide adds a path from the second register output to data memory input and disables register-file write enable.

### Simplified explanation

For sw, the CPU:

- computes address from rs + imm,

- takes data from register rt,

- writes it into memory.

### Important difference from lw

- lw reads memory and writes a register

- sw reads a register and writes memory

So the write-enable targets change.

## Slide 21 – Fifth Instruction: beq $1, $2, target

This slide adds:

- shift-left-by-2 unit,

- branch target adder,

- and a mux to select the next PC.

### Simplified explanation

For beq, the CPU must:

- compare two registers,

- decide if they are equal,

- if equal, branch to a computed target,

- otherwise continue with PC + 4.

### Why shift left by 2?

Because the branch immediate represents word offset, but addresses are in bytes.

### Main datapath idea

Now the PC no longer always takes PC + 4. It may take a branch target instead.

## Slide 22 – Sixth Instruction: j

This slide adds:

- a shifter for the 26-bit jump immediate,

- and another mux on the PC input for jump target.

### Simplified explanation

A jump instruction changes control flow directly.

### What hardware is needed

The CPU must build the jump target address and send it to the PC.

### Student takeaway

Both beq and j affect the PC, but in different ways:

- beq is conditional,

- j is unconditional.

## Slide 23 – Seventh, Eighth, Ninth Instructions

This slide discusses sll, slt, jal, and jr, explaining what extra paths they would need.

### Simplified explanation

This slide teaches a powerful design principle:

Every extra instruction may need extra datapath support.

### Examples

- sll needs a shifter,

- slt needs comparison-result handling,

- jal needs to save PC + 4 in $ra,

- jr needs a path from register value to PC.

### Revision point

Understanding one instruction at a time helps students see why datapath complexity grows.

## Slide 24 – Clock Timing

This slide explains timing concerns:

- avoiding races,

- avoiding simultaneous read/write issues,

- keeping the clock simple and global,

- separating PC changes from other logic if necessary.

### Simplified explanation

Even if the datapath connections are correct, bad timing can still break the processor.

### Why timing matters

If signals change too early or too late:

- the wrong instruction may be read,

- unstable values may appear,

- behavior becomes unpredictable.

### Student summary

Hardware correctness depends on both:

- logic,

- and timing.

## Slide 25 – This Unit: Processor Design

This slide repeats the unit map, now emphasizing control.

### Simplified explanation

The lecture is now shifting from datapath construction to the signals that control the datapath.

## Slide 26 – What Is Control?

This slide says the simple datapath is governed by 9 control signals, including mux selectors and write-enable signals.

### Simplified explanation

Control tells the datapath:

- which path to use,

- which component should write,

- what operation the ALU should do,

- whether to branch or jump.

### Key student idea

The datapath is the hardware possibility.
Control selects the correct possibility for the current instruction.

## Slide 27 – Example: Control for add

This slide shows the specific control settings for add, including values for Rwe, Rdst, ALUinB, ALUop, DMwe, Rwd, BR, and JP.

### Simplified explanation

For add:

- register writing is enabled,

- destination is rd,

- ALU uses two register inputs,

- memory is not written,

- no branch or jump occurs.

### Student takeaway

add activates only the paths needed for register arithmetic.

## Slides 28–29 – Example: Control for sw

These slides show the control settings for sw, including:

- no register write,

- ALU input from immediate,

- memory write enabled,

- branch/jump disabled,

- some signals are don’t-care (X).

### Simplified explanation

For sw, the CPU needs:

- address calculation,

- data from register,

- write to memory.

It does not need register write-back.

### What “don’t care” means

A signal is don’t-care when its value does not affect the outcome for that instruction.

## Slide 30 – Example: Control for beq

This slide shows control differences for a branch:

- register write disabled,

- memory write disabled,

- ALU used for comparison,

- branch enabled.

### Simplified explanation

For beq, the key job is not arithmetic result storage. The key job is decision-making.

The CPU uses the ALU to compare operands and then decides whether to update the PC with the branch target.

## Slide 31 – How Is Control Implemented?

This slide asks where the control signals come from in the first place. It shows the datapath with a blank control block.

### Simplified explanation

This is the central control-design question:

How do instruction bits become control signals?

That is what the next slides answer.

## Slide 32 – Implementing Control

This slide explains:

- each instruction has a unique control pattern,

- most signals are functions of opcode,

- some may be encoded directly in the instruction,

- control can be implemented with a lookup table or combinational logic.

### Simplified explanation

The control unit reads the instruction and converts it into signals.

### Two big methods

- Table lookup

- Logic equations

Different hardware designers choose based on complexity, speed, and design goals.

## Slide 33 – Control Implementation: ROM

This slide presents ROM-based control, where opcode indexes rows and output bits are the control signals. It includes a table for add, addi, lw, sw, beq, and j.

### Simplified explanation

ROM control works like a dictionary:

- input: opcode

- output: ready-made control bits

### Why students like this model

It is easy to understand because it is direct and visual.

## Slide 34 – ROM vs. Combinational Logic

This slide says ROM works for small examples, but real processors have far more instructions and control signals, making fast ROM-based control harder. It suggests combinational logic as an alternative.

### Simplified explanation

ROM is simple but may become too slow or too large.
Combinational logic is often faster and better for large, real machines.

### Key lesson

Control must be fast, because if control is slow, the whole processor becomes slow.

## Slide 35 – Control Implementation with Decoder

This slide shows a one-hot decoder style combinational logic implementation for control.

### Simplified explanation

Instead of storing control patterns in ROM, we can build logic that directly generates the required signals.

### Student focus

You do not need to memorize every wire here. Just understand:

- instruction decoding can activate one control path,

- logic gates then produce the needed outputs.

## Slide 36 – This Unit: Processor Design

This slide shifts attention to exceptions.

### Simplified explanation

The lecture now moves from normal instruction execution to unusual events that interrupt normal flow.

## Slide 37 – Exceptions

This slide lists examples of exceptions and interrupts and explains that they are infrequent events requiring OS handling. It distinguishes between events that end the program and those that are fixed and then execution resumes.

### Simplified explanation

An exception is something unusual, such as:

- divide-by-zero,

- illegal instruction,

- page fault,

- timer interrupt.

### Big idea

The processor cannot just ignore these. It must pause normal execution and hand control to the operating system.

## Slide 38 – Exception Handling

This slide explains the software view:

- control transfers to an exception handler,

- OS gets access to privileged registers,

- the cause and return PC are stored,

- OS handles the event,

- control returns to user code,

- and SYSCALL uses the same general mechanism.

### Simplified explanation

When an exception happens:

- processor stops normal program,

- jumps to OS handler,

- OS inspects the problem,

- OS decides what to do,

- execution may resume later.

## Slides 39–40 – MIPS Exception Handling

These slides describe special MIPS registers such as EPC, exception type, bad virtual address, and exception mask, and mention privileged instructions like mfc0, mtc0, and rfe. One of the slides explicitly tells students not to obsess over exact MIPS details and instead focus on the big picture.

### Simplified explanation

For student understanding, the big picture matters more than register names.

### Big picture

A processor must:

- save enough state,

- identify the problem,

- let the OS handle it,

- restore execution if possible.

That is the real lesson.

## Slide 41 – Implementing Exceptions

This slide explains why architects care about exceptions:

- the datapath and control must support them,

- including recognition, transfer of control, and privileged OS mode.

### Simplified explanation

Exceptions are not only an operating-system concept. Hardware must also be designed to support them.

That means processor design includes:

- normal instruction execution,

- and abnormal event handling.

## Slide 42 – Datapath with Support for Exceptions

This slide shows an extended datapath that includes coprocessor registers and PSR-related support. It notes that the PSR indicates whether the processor is in privileged mode.

### Simplified explanation

This is a more advanced datapath showing how exception support is added.

### Student focus

Do not try to memorize all lines. Understand the purpose:

- save exception information,

- support privileged mode,

- route control into OS handling when required.

## Slide 43 – Summary

This slide says we now know how to build a fully functional processor, but memory is still treated as a black box and the processor is still very slow.

### Simplified explanation

At this stage, the design is functionally complete, but not yet efficient.

This is normal in architecture:

- first make it work,

- then make it fast.

## Slide 44 – “Single-Cycle” Performance

This slide explains:

- single-cycle CPU has CPI = 1,

- but it is slow because the clock period must match the longest instruction,

- and in this datapath lw is the longest because it passes through multiple structures, especially slow data memory.

### Simplified explanation

Single-cycle seems attractive because:

- one instruction = one cycle

But the problem is:

- every instruction must wait for the slowest one.

### Why lw is slow

It goes through:

- instruction memory

- register file read

- ALU

- data memory

- register file write

That long path forces a slow clock.

### Student lesson

Low CPI does not always mean high performance.

## Slide 45 – Next up: Memory Systems

This closing slide says the next topic is memory systems.

### Simplified explanation

This lecture ends by preparing students for the next major topic:

- now that we know the processor structure,

- we must understand memory better,

- because memory strongly affects performance.

# Ultra-Short Revision Version

## Slides 1–4

Introduction, course position, unit scope, reading.

## Slides 5–6

CPU is more than an ALU. Datapath and control are the two key ideas.

## Slides 7–8

Processor design starts from the ISA and selected representative instructions.

## Slides 9–14

Hardware building blocks: register, register file, memory, ALU, mux, adders, extenders.

## Slides 15–24

Build datapath step by step for:

- add

- addi

- lw

- sw

- beq

- j
and discuss timing.

## Slides 25–35

Understand control signals and control implementation:

- what control is,

- control examples,

- ROM vs logic.

## Slides 36–42

Exceptions:

- what they are,

- why they matter,

- how processor and OS handle them.

## Slides 43–45

Summary and performance:

- single-cycle works,

- but it is slow,

- next topic is memory systems.