# Computer Architecture – Week 6 Handout

# Pipelining Basics

## Student Notes for Lecture Understanding and Exam Preparation

# 1. Introduction

Up to this point, students have studied:

- Instruction Set Architecture (ISA),

- datapath,

- control,

- and how one instruction moves through the processor.

That is important, but it describes instruction execution mostly in a simple and direct way. The next major question in computer architecture is:

## How can a processor execute instructions more efficiently without waiting for one complete instruction to fully finish before starting the next one?

The answer is pipelining.

Pipelining is one of the most important concepts in computer architecture because it explains how processors improve performance by overlapping the execution of multiple instructions.

This topic takes students from the idea of single-instruction execution to the much more powerful idea of instruction overlap.

# 2. Why Pipelining is Needed

Imagine a processor that completes one full instruction before starting the next one.

That means:

- instruction 1 completes fully,

- then instruction 2 begins,

- then instruction 3 begins,

- and so on.

This works, but it is inefficient.

Why?

Because different hardware parts of the CPU are not being used all the time. Some parts stay idle while others are working.

For example:

- while one instruction is being decoded, the fetch unit may be idle,

- while one instruction is using the ALU, another instruction could have been fetched,

- while one instruction writes back a result, another could already be halfway through execution.

So the problem is not that the processor cannot execute instructions. The problem is that it may not be using its hardware efficiently.

Pipelining solves this inefficiency.

# 3. Basic Idea of Pipelining

Pipelining means dividing instruction execution into stages and allowing different instructions to occupy different stages at the same time.

Instead of treating one instruction as one large complete task, the processor divides its execution into smaller steps.

Then:

- one instruction may be in the fetch stage,

- another in the decode stage,

- another in the execute stage,

- and another in the memory stage,

all at the same time.

This means multiple instructions are “in progress” together.

# 4. Everyday Analogy for Pipelining

A useful analogy is a laundry process or factory assembly line.

Suppose washing clothes involves these steps:

- Wash

- Dry

- Fold

If one person does all three for one batch before starting the next, total work takes longer.

But if the process is arranged like an assembly line:

- batch 1 may be drying,

- while batch 2 is washing,

- and batch 3 is being prepared,

then more work gets done in the same amount of time.

The same logic applies to pipelining in processors.

The goal is not necessarily to make one individual instruction much shorter. The goal is to make the overall flow of instructions faster.

# 5. Pipelining and CPU Performance

Pipelining improves throughput.

Throughput means:

## how many instructions are completed in a certain amount of time

Pipelining allows the processor to complete instructions more frequently because multiple instructions move through the machine together.

This does not automatically mean that each single instruction becomes faster from start to finish.

That distinction is very important.

# 6. Throughput vs Latency

Students often confuse these two terms, so they must be understood clearly.

## Latency

Latency is the total time taken by one instruction from start to finish.

## Throughput

Throughput is the rate at which instructions are completed.

### Important idea

Pipelining mainly improves throughput, not necessarily latency.

A single instruction may still need several stages to complete, but because many instructions overlap, the processor completes more instructions over time.

So pipelining is about making the processor more productive, not magically making every individual instruction instant.

# 7. Pipeline Stages

A pipeline works by breaking instruction execution into stages.

A common five-stage pipeline model includes:

- Instruction Fetch (IF)

- Instruction Decode (ID)

- Execute (EX)

- Memory Access (MEM)

- Write Back (WB)

This is one of the most widely used introductory pipeline models in computer architecture.

# 8. Instruction Fetch (IF)

In the fetch stage, the processor gets the instruction from instruction memory using the program counter.

This stage answers:

- what is the next instruction?

- where is it located in memory?

The PC is used to find the instruction, and then the instruction is brought into the processor.

At the same time, the processor prepares for the next sequential instruction address.

# 9. Instruction Decode (ID)

In the decode stage, the processor interprets the fetched instruction.

This means it determines:

- what operation the instruction represents,

- which registers are involved,

- whether an immediate value is present,

- what control signals will be needed.

The decode stage often also includes reading source register values from the register file.

So decode is where the instruction begins to become meaningful to the processor.

# 10. Execute (EX)

In the execute stage, the processor performs the main computation.

Depending on the instruction, this may include:

- arithmetic operations,

- logical operations,

- effective address calculation,

- comparison for branching.

For example:

- an add instruction performs addition here,

- a lw may calculate the memory address here,

- a branch may compare values here.

The ALU plays a central role in this stage.

# 11. Memory Access (MEM)

In the memory stage, instructions that involve memory interact with data memory.

This stage is important for:

- lw (load word),

- sw (store word),

- and sometimes branch-related operations depending on design.

Not every instruction truly needs memory access in the same way, but the pipeline stage exists so that memory-related work can be handled systematically.

For an arithmetic instruction like add, this stage may not do significant memory work, but the instruction still passes through the pipeline structure.

# 12. Write Back (WB)

In the write-back stage, the processor writes the final result into the destination register if needed.

For example:

- an add writes the ALU result into a register,

- a lw writes the loaded memory value into a register.

After this stage, the instruction is considered fully completed.

# 13. What Happens Without Pipelining

Without pipelining, one instruction passes through all stages completely before the next instruction begins.

So the execution pattern looks like this:

- Instruction 1 does IF, ID, EX, MEM, WB

- then Instruction 2 starts

- then Instruction 3 starts

This means many hardware units remain unused during different moments.

For example:

- while instruction 1 is in EX, the fetch hardware is not doing useful work for another instruction,

- while instruction 1 is in WB, the decode stage may be idle.

That is wasteful.

# 14. What Happens With Pipelining

With pipelining, the stages overlap.

A simplified flow looks like this:

- Cycle 1: Instruction 1 in IF

- Cycle 2: Instruction 1 in ID, Instruction 2 in IF

- Cycle 3: Instruction 1 in EX, Instruction 2 in ID, Instruction 3 in IF

- Cycle 4: Instruction 1 in MEM, Instruction 2 in EX, Instruction 3 in ID, Instruction 4 in IF

- Cycle 5: Instruction 1 in WB, Instruction 2 in MEM, Instruction 3 in EX, Instruction 4 in ID, Instruction 5 in IF

Now the processor is using different parts of its hardware much more efficiently.

This is the key practical benefit of pipelining.

# 15. Why Pipelining Increases Efficiency

Pipelining increases efficiency because the processor is no longer waiting for one instruction to finish before beginning the next one.

Instead:

- fetching can happen while another instruction is being decoded,

- decoding can happen while another is being executed,

- writing back can happen while new instructions are already entering the machine.

This means the CPU hardware is working more continuously.

In computer architecture, good performance often comes not only from making components powerful, but from using them efficiently and continuously.

Pipelining is one of the clearest examples of that idea.

# 16. Pipeline Registers

Between stages, the processor needs temporary storage to hold intermediate results and control information.

These temporary storage elements are called pipeline registers.

For example:

- after fetch, the instruction must be held before decode,

- after decode, operand values and control information must be held before execute,

- after execute, results must be preserved before memory stage,

- and so on.

Without these intermediate registers, the stage outputs would not be safely transferred from one stage to the next.

Pipeline registers are therefore essential to make a pipeline work correctly.

# 17. Clocking in a Pipeline

A pipelined processor usually advances instructions stage by stage using the clock.

At each clock cycle:

- one stage completes its current work,

- results move into the next pipeline register,

- and each instruction advances one step forward.

This means the pipeline has a regular rhythm.

The clock helps coordinate the movement of instructions through the stages so that the whole processor behaves in an orderly way.

So in a pipelined design, timing becomes even more important than before.

# 18. Ideal Pipeline Behavior

In an ideal pipeline:

- one new instruction enters the pipeline every cycle,

- one instruction completes every cycle after the pipeline is filled.

This is the dream scenario.

At the beginning, the pipeline needs a few cycles to fill. But once it is full, the processor can complete instructions at a much higher rate than a non-pipelined system.

This is why pipelining is so powerful in theory.

# 19. Pipeline Fill and Drain

A pipeline does not become fully productive instantly.

At the start:

- the pipeline must fill stage by stage.

At the end of a program or code section:

- the remaining instructions still need time to leave the pipeline.

These effects are called:

- pipeline fill

- pipeline drain

So although the steady-state performance can be excellent, the beginning and end do not operate at full ideal efficiency.

This is normal and important for students to understand.

# 20. Speedup from Pipelining

Pipelining improves performance mainly by overlapping work.

If instruction execution is divided into several balanced stages, then in theory the speedup may approach the number of pipeline stages.

For example, a 5-stage pipeline may ideally approach a speedup near 5 compared to a fully non-pipelined version.

However, this ideal is rarely achieved exactly in real systems because:

- stages may not be perfectly balanced,

- some instructions behave differently,

- and hazards or delays may occur.

Still, pipelining usually provides major performance improvement.

# 21. Balanced vs Unbalanced Pipeline Stages

Pipelining works best when the stages take roughly similar amounts of time.

If one stage is much slower than the others, then the clock period must be long enough for that slowest stage.

That means the whole pipeline becomes limited by the slowest stage.

So a good pipeline design tries to balance stage delays.

This is an important architecture principle:

## the slowest stage often determines the speed of the entire pipeline

# 22. Why Pipelining Does Not Solve Everything

Students should not think pipelining is a perfect solution to all performance problems.

Pipelining improves instruction overlap, but it introduces new challenges.

For example:

- instructions may depend on one another,

- branches may disrupt the smooth flow,

- memory access may still be slow,

- pipeline stages may interfere with each other.

These problems are called pipeline hazards, and they are the natural next topic after pipelining basics.

So this week focuses on the basic concept and benefit of pipelining. The complications come next.

# 23. Example of Pipeline Thinking

Consider these three instructions:

add  $t0, $t1, $t2
sub  $t3, $t4, $t5
and  $t6, $t7, $t8

In a non-pipelined design, the processor may fully finish add before starting sub, and then fully finish sub before starting and.

In a pipelined design:

- while add is in EX,

- sub may already be in ID,

- and and may already be in IF.

This does not mean the instructions are mixed incorrectly. It means the processor is using separate stages efficiently at the same time.

That is the essence of pipelining.

# 24. Practical Understanding of Pipelining

Students often wonder whether pipelining exists only in textbooks. It does not.

Pipelining is a real and central design idea used in processors because it allows better use of CPU hardware.

Whenever a processor is designed to handle instruction flow efficiently across stages, pipeline thinking is involved.

In real systems, pipelines can be:

- simple,

- deep,

- or highly advanced.

But the basic logic remains the same:

## overlap the execution of multiple instructions

# 25. Common Student Confusions

One common confusion is thinking that pipelining means one instruction is split across many different processors. That is not correct.

Pipelining means one processor is divided into stages, and multiple instructions occupy different stages at the same time.

Another confusion is thinking that pipelining reduces the number of steps in instruction execution. It does not. The same logical work still exists. Pipelining simply overlaps the work better.

Another confusion is assuming that pipelining always makes every program perfectly faster. In reality, the effect depends on:

- instruction flow,

- dependencies,

- branch behavior,

- and hardware design.

So pipelining improves potential performance, but actual performance depends on many conditions.

# 26. Link to Earlier Topics

Pipelining is closely connected to the earlier lectures.

## Connection with ISA

The instruction set determines what kinds of instructions flow through the pipeline.

## Connection with datapath

The datapath stages become the natural basis for the pipeline stages.

## Connection with control

Control signals must move properly through the pipeline and guide each instruction at each stage.

So pipelining is not a separate unrelated idea. It builds directly on everything already studied.