# Week 8 Handout

# Performance Enhancement and Instruction-Level Parallelism

## Student Notes for Lecture Understanding and Exam Preparation

# 1. Introduction

In the previous weeks, students studied:

- how instructions are executed,

- how pipelining improves throughput,

- and how hazards reduce ideal pipeline performance.

Now the next major question is:

## How do architects measure processor performance, and how do they improve it beyond basic pipelining?

This week focuses on performance enhancement, especially through a very important idea called Instruction-Level Parallelism (ILP).

This topic is central to computer architecture because it connects:

- processor design,

- pipelining,

- hazards,

- and real performance improvement.

It helps students move from simply understanding processor structure to understanding why one architecture performs better than another.

# 2. What Performance Really Means

When people say that one processor is “faster” than another, that does not simply mean it has a higher clock speed.

Real processor performance depends on several interacting factors, such as:

- how many instructions a program needs,

- how many cycles each instruction takes on average,

- and how long each cycle lasts.

So performance is not one single number. It is the result of several architectural decisions working together.

This is why computer architects use more formal ways to study performance.

# 3. CPU Performance Formula

A common way to understand CPU performance is through the idea that:

## CPU Time = Instruction Count × CPI × Clock Cycle Time

This is one of the most important performance ideas in computer architecture.

It means total execution time depends on:

- Instruction Count (IC): how many instructions the program executes

- CPI: cycles per instruction on average

- Clock Cycle Time: how long one cycle takes

This formula is powerful because it shows that performance can be improved in more than one way.

# 4. Instruction Count

Instruction count means the total number of instructions needed to run a program.

Different processors or different instruction set designs may require:

- more instructions,

- or fewer instructions,
for the same task.

Instruction count is affected by:

- ISA design,

- compiler quality,

- program structure,

- and sometimes architectural features.

A processor may have a fast clock, but if it needs many more instructions to perform the same work, its overall performance may still suffer.

So instruction count is one major part of performance.

# 5. CPI – Cycles Per Instruction

CPI means the average number of clock cycles each instruction takes.

In an ideal simple pipeline, students often imagine:

- one instruction completed every cycle,

- so CPI seems close to 1.

But in real processors, hazards, memory delays, branches, and other effects can increase CPI.

So CPI reflects how efficiently the processor is actually running.

A low CPI is usually good, but it must be considered together with instruction count and clock cycle time.

# 6. Clock Cycle Time

Clock cycle time is the length of one processor cycle.

If cycle time is smaller, the processor can run more cycles per second.

However, reducing cycle time is not always easy, because the cycle must still be long enough for the hardware stage work to complete properly.

A processor may have:

- a low CPI,

- but very long cycle time,
and still perform poorly.

So clock rate alone is not enough to judge architecture quality.

# 7. Why One Performance Factor Alone Is Not Enough

Students often assume that:

- higher GHz means better performance,

- or lower CPI means better performance.

But this is incomplete.

For example:

- a processor may have a very high clock speed but poor CPI,

- another may have better CPI but require more instructions,

- another may reduce instruction count but need longer cycle time.

This means performance is always about balance.

Computer architecture is the art of improving overall performance, not just one isolated metric.

# 8. How Pipelining Affects Performance

Pipelining improves performance mainly by increasing throughput and reducing the average time between completed instructions.

In ideal conditions, pipelining allows the processor to complete instructions more frequently.

That can help reduce effective CPI toward 1.

However, real performance depends on:

- hazards,

- stalls,

- branch disruptions,

- memory delays,

- and stage balance.

So pipelining provides the opportunity for high performance, but actual performance depends on how well the pipeline is managed.

# 9. Pipeline Efficiency

Pipeline efficiency means how close the real pipeline comes to its ideal behavior.

In an ideal pipeline:

- one instruction completes every cycle after filling,

- hardware is always busy,

- and CPI stays near 1.

In a real pipeline:

- stalls may appear,

- bubbles may be inserted,

- branches may disrupt flow,

- and some instructions may take more effort than others.

So pipeline efficiency depends on how much useful work is actually completed relative to the pipeline’s theoretical capability.

# 10. What Reduces Pipeline Efficiency

Several things reduce the ideal benefit of pipelining:

- structural hazards

- data hazards

- control hazards

- load-use delays

- branch penalties

- memory stalls

- uneven stage delays

These factors increase CPI above the ideal value.

So when architects try to improve performance, they often focus on reducing these inefficiencies.

This is where performance enhancement techniques become important.

# 11. Instruction-Level Parallelism (ILP)

Instruction-Level Parallelism means the extent to which instructions in a program can be executed in overlapping or parallel fashion.

In simple words, ILP asks:

## How many instructions can potentially make progress at the same time?

Pipelining is one form of ILP, because different instructions occupy different stages at once.

But ILP goes beyond basic pipelining. It includes the larger idea of finding and exploiting independence between instructions so that more work can happen together.

This is one of the central performance ideas in modern processor architecture.

# 12. Why ILP Matters

Programs are made of many instructions, but not all instructions are equally dependent on each other.

Some instructions must wait because they need earlier results.
Other instructions are independent and could potentially proceed without waiting.

If the processor can identify and exploit these independent instructions, it can perform more work in the same time.

That is why ILP matters:

- it helps increase throughput,

- it improves hardware utilization,

- and it is one of the main ways processors become faster.

# 13. Dependent and Independent Instructions

To understand ILP, students must distinguish between dependent and independent instructions.

## Dependent instructions

A dependent instruction needs the result of an earlier instruction.

Example:

add $t0, $t1, $t2
sub $t3, $t0, $t4

Here the second instruction depends on the result of the first.

## Independent instructions

An independent instruction does not need the result of the earlier one.

Example:

add $t0, $t1, $t2
and $t5, $t6, $t7

These may potentially progress in overlapping ways.

The more independent instructions a processor can exploit, the more ILP it can use.

# 14. ILP and Pipeline Performance

Basic pipelining already overlaps instructions stage by stage, but ILP asks whether even more overlap can be achieved.

If instructions are independent:

- the processor may keep the pipeline full more easily,

- hazards may be fewer,

- and performance may improve.

If instructions are heavily dependent:

- the pipeline may stall frequently,

- forwarding may only help partially,

- and CPI may increase.

So ILP helps explain why some code runs very efficiently while other code does not.

# 15. Sources of Limited ILP

Instruction-level parallelism is powerful, but it is not unlimited.

Several things restrict it:

- data dependencies between instructions

- control dependencies from branches

- limited hardware resources

- memory access delays

- hazards and stalls

This means that even if a processor is designed to exploit ILP, the actual program may not always provide enough independent instructions to keep the machine fully busy.

So ILP is partly an architectural issue and partly a program behavior issue.

# 16. Performance Enhancement by Reducing CPI

One way to improve processor performance is to reduce CPI.

This can happen by:

- improving hazard handling,

- increasing forwarding effectiveness,

- reducing branch penalties,

- improving memory hierarchy,

- and keeping the pipeline more continuously active.

A lower CPI means fewer wasted cycles per instruction on average.

So many architectural improvements are really attempts to bring actual CPI closer to ideal CPI.

# 17. Performance Enhancement by Reducing Clock Cycle Time

Another path to better performance is reducing clock cycle time.

This often requires:

- shorter or better-balanced pipeline stages,

- faster hardware paths,

- better circuit design,

- and reduced stage delay.

However, this also creates trade-offs.

If the pipeline becomes too deep:

- more stages may increase control complexity,

- branch penalties may become worse,

- hazard handling may become harder.

So making cycles shorter is useful, but not cost-free.

# 18. Performance Enhancement by Reducing Instruction Count

Performance can also improve if the processor or software reduces the total number of instructions needed.

This may happen through:

- better compiler optimization,

- more expressive instructions,

- or better code scheduling.

However, reducing instruction count may sometimes require:

- more complex instructions,

- longer cycle time,

- or more difficult hardware.

So again, performance design is about balance.

# 19. Why Speedup Is Used

Architects often compare designs using speedup.

Speedup tells us how much faster one design is compared to another.

The idea is simple:

## Speedup = Old Execution Time / New Execution Time

If the new design finishes the same job in less time, speedup is greater than 1.

Speedup is useful because it focuses on real performance improvement, not just on one internal metric like clock speed or CPI.

# 20. Ideal vs Real Speedup

Students should understand that theoretical speedup and real speedup are often different.

A technique may promise large speedup in theory, but real systems are affected by:

- hazards,

- memory behavior,

- branch delays,

- unbalanced stage timing,

- and limited instruction independence.

So real performance analysis must always ask:

## How much of the theoretical gain is actually achieved in practice?

That question is one of the most important habits in computer architecture thinking.

# 21. Superscalar Idea – A Step Beyond Basic Pipelining

One major performance-enhancement idea is the superscalar processor.

A superscalar design attempts to issue and execute more than one instruction per cycle.

This goes beyond simple pipelining.

Basic pipelining overlaps stages across instructions.
Superscalar design tries to let multiple instructions begin or proceed in parallel within the same cycle.

This is a more advanced use of ILP.

It depends on:

- hardware capability,

- instruction independence,

- and good scheduling.

# 22. Why Superscalar Design Is Powerful

If the processor can safely issue two or more independent instructions in one cycle, throughput can improve significantly.

But this requires:

- more hardware resources,

- better hazard checking,

- dependency handling,

- and smarter control logic.

So superscalar design shows that higher performance often requires not just faster components, but more complex organization.

This idea becomes easier to appreciate after students already understand pipelining and hazards.

# 23. Instruction Scheduling

Instruction scheduling means arranging instructions in an order that improves pipeline use and reduces stalls.

This may be done by:

- the compiler,

- the hardware,

- or both.

The goal is to place independent instructions between dependent ones so the processor has useful work to do while waiting for results.

This helps exploit ILP and improve performance.

So scheduling is an important performance-enhancement idea even if the hardware itself does not change drastically.

# 24. Compiler and Architecture Relationship

Performance is not created by hardware alone.

The compiler also affects performance because it can influence:

- instruction ordering,

- register usage,

- dependency exposure,

- and the total instruction sequence presented to the CPU.

A strong architecture and a good compiler often work together.

This is why architecture and software optimization are deeply connected.

The processor may provide opportunities for high ILP, but the code must also expose enough useful instruction behavior.

# 25. Memory and Performance Enhancement

Even when pipeline organization is strong, memory can still limit performance.

For example:

- cache misses may stall instruction flow,

- memory latency may reduce effective throughput,

- load-use delays may persist,

- and branch-heavy code may reduce available ILP.

So performance enhancement must not focus only on CPU core logic. It must also consider the memory system.

This is why architecture topics are interconnected. CPU and memory performance cannot be separated completely.

# 26. Practical Limits on ILP

In theory, students may imagine that processors could keep executing more and more instructions together.

But ILP has practical limits.

These limits include:

- true data dependencies,

- branch uncertainty,

- limited hardware width,

- memory bottlenecks,

- and the complexity cost of checking many instructions together.

So ILP is valuable, but it cannot grow infinitely.

This is one reason modern architecture also moved toward:

- multicore designs,

- thread-level parallelism,

- and other broader forms of performance improvement.

# 27. Why CPI Alone Is Not Enough

Students must now think more carefully than before.

A design with low CPI may still perform poorly if:

- clock cycle time is too long,

- instruction count is too high,

- memory penalties dominate,

- or the workload does not match the architecture well.

Similarly, a design with more advanced features may not always perform better on every type of program.

So architectural performance is always contextual. It depends on the balance between:

- workload,

- hardware,

- control,

- memory,

- and instruction behavior.

# 28. Practical Example of ILP Thinking

Consider these instructions:

add $t0, $t1, $t2
and $t3, $t4, $t5
sub $t6, $t7, $t8

These instructions are mostly independent of one another.

That means a processor with good ILP capability may overlap or even issue them in highly efficient ways.

Now compare with:

add $t0, $t1, $t2
sub $t3, $t0, $t4
and $t5, $t3, $t6

This second sequence contains more dependencies.

So even if the processor is powerful, the available ILP is lower because the instructions rely on one another.

This shows that ILP depends on both:

- the architecture,

- and the code structure.

# 29. Relationship Between ILP and Hazards

Hazards reduce the processor’s ability to exploit ILP.

For example:

- data hazards prevent dependent instructions from advancing freely,

- control hazards disturb future instruction flow,

- structural hazards reduce simultaneous hardware use.

So hazard handling is directly connected to ILP.

A processor with better hazard handling can often exploit more instruction-level parallelism.

That is why Week 7 and Week 8 are tightly linked.

# 30. Why Performance Enhancement Is a Core Architecture Topic

This topic is central to computer architecture because it teaches students how to think like architects.

Instead of only asking:

- what does this instruction do?

students now ask:

- how efficiently is the hardware being used?

- what limits throughput?

- what increases CPI?

- what architectural features improve real execution time?

That is the heart of architecture thinking:

## understanding performance as the result of structured design choices

# 31. Common Student Confusions

One common confusion is assuming that faster clock speed automatically means better processor performance. This is not always true.

Another confusion is assuming that pipelining alone guarantees full performance gain. In reality, hazards and dependencies limit ideal behavior.

Another confusion is thinking that ILP means all instructions can run in parallel. That is also not true. Only instructions without harmful dependencies can safely overlap more aggressively.

Students should remember:

- performance is limited by many factors,

- and architecture is about improving the overall system, not just one part.